General Keywords

Multiplexer: A digital component  that uses a selector bit to select one of two outputs.26

Assembler: A program that converts source-code programs from assembly language to machine language.26

Linker: A companion program of the assembler that combines the individual files created by an assembler into a single executable program.26

Debugger: A third program that provides a way for a programmer to trace the execution of a program and examine the contents of memory.26

Registers: Storage locations contained in CPU.33

Clock: synchronizes the internal operations of CPU with other system components.33

Control Unit: Coordinates the sequencing of steps involved un executing machine instructions.33

Arithmetic logic unit: The ALU performs arithmetic operations such as addition and subtraction and logical operations such as AND,OR and NOT.33

Memory Storage Unit: It's  where the instructions and data are held while a computer program is running. It receives requests for data from CPU, transfers data from random access memory (RAM) to CPU.All processing of data takes place within CPU, so programs residing in memory must be copied into the CPU

before they can execute.Individual program instructions can be copied into CPU one at a time.or groups of instructions can be copied together. 33

Bus: is a group of parellel wires that transfer data from one part of the computer to another.33

Databus: Transfers instructions  and data between the CPU and memory.33

I/O bus: Transfers data between the CPU and the system input/output devices.34

Control bus: Uses binary signals to synchronize actions of all devices attached to the system bus.34

Address bus: Holds the addresses of instructions and data when the currently

executing instruction transfers data between the CPU and memory.34

Clock: Each operation involving the CPU and the system bus is synchronized by an internal clock p7lsing at a constant rate.34

Machine/Clockcycle: The basic unit of time for machine instructions. The length of a clock cycle is the time required for one complete clock pulse which is depicted as the time between one falling edge and the next.34

Instruction execution cycle: Before the execution of a program CPU goes through this sequence of steps.34

Operand: An operand is a value that is either an input or an output to an

operation.35

Cache:  It's a high speed memory used to store the recently used  instructions and data.36

Cache Hit: When the processor is able to find its data in cache memory we call it Cache Hit.36

Cache Miss: If the CPU tries to find something in cache but it's  not there, we call that a cache hit.36

Protected Mode: It's  the native state of the processor in which all instructions and features are available. Programs are given separate memory areas called segments and processor prevents program s from referencing programs out of their assigned segments.38

Real Address Mode: Implements the programming environment of an early Intel processor  with a few extra features, such as the ability to switch into other modes.This mode is useful if a program requires direct access to system memory and hardware devices.38

Virtual - 8086 Mode: While in protected mode , the processor  can directly  execute real-address mode software such as MS-DOS programs in a safe environment. If program crashes or attemps to write data into system memory area it would not affect other programs running at the same time. A modern OS can execute multiple separate virtual 8086 sessions at the same time.38

System Management Mode: System Management Mode or SMM  provides an OS with a mechanism for implementing functions such as power management and system security.These functions are usually implemented by computer  manufacturers who customize the processor for a particular system setup.38

Registers : High speed storage locations directly inside CPU, designed to be accessed at much higher speed than conventional memory.It should not be used for ordinary arithmetic or data transfer except at an advanced level of programming. It is often called the extended frame pointer register.39

=====================================================================